# Engineering Report

Project Title:
Arduino R4 WiFi–Based Security System with PIR Motion Detection and Web-Controlled Arm/Disarm

Course: Engineering / Embedded Systems
Author: Dante D’Abramo
Date: 2026-01-30

## Abstract

This report presents the design, implementation, and validation of a network-enabled security system based on the Arduino R4 WiFi platform. The system employs a passive infrared (PIR) motion sensor for intrusion detection and provides visual and audible alerts through LED indicators and a piezoelectric buzzer. A web-based interface hosted on the Arduino R4 WiFi allows remote arming and disarming of the system. The project demonstrates a structured engineering approach, integrating hardware design, firmware architecture, and validation testing suitable for academic and professional prototyping contexts.

## 1. Introduction

Security systems require accurate sensing, reliable alert mechanisms, and remote control capabilities. Advances in embedded platforms now allow these features to be implemented on a single development board. The Arduino R4 WiFi integrates a Renesas RA4M1 microcontroller with an ESP32-S3 Wi-Fi coprocessor, enabling real-time sensor processing alongside network connectivity. This project leverages these capabilities to create a compact, modular, and scalable motion-detection security system.

## 2. System Requirements

Table 1 – Functional System Requirements

| ID | Requirement Description |
| --- | --- |
| R1 | Detect human motion using a PIR sensor |
| R2 | Provide visual indication of system state |
| R3 | Generate an audible alarm when motion is detected |
| R4 | Allow remote arming and disarming via Wi-Fi |
| R5 | Operate using minimal external components |

## 3. Components

Table 2 – Bill of Materials (BOM)

| Component | Quantity | Description |
| --- | --- | --- |
| Arduino R4 WiFi | 1 | Main control and networking board |
| PIR Motion Sensor | 1 | Motion detection |
| 3.0 mm LED | 1 | Armed/status indicator |
| 5.0 mm LED | 1 | Alarm indicator |
| Active Piezo Buzzer | 1 | Audible alarm |
| Jumper Wires | Assorted | Electrical connections |
| Enclosure | 1 | Mechanical protection |

## 4. System Architecture

The system is divided into sensing, control, and communication layers. The RA4M1 microcontroller handles real-time input from the PIR sensor and drives the output devices. The ESP32-S3 coprocessor hosts a lightweight HTTP server that receives user commands over Wi-Fi.

Figure 1 – High-Level System Architecture
PIR Sensor → Microcontroller Logic → LEDs / Buzzer
Web Client ↔ Wi-Fi Interface ↔ Control State

## 5. Hardware Design

### 5.1 Electrical Connections

Table 3 – Pin Assignment Summary

| Signal | Arduino R4 WiFi Pin | Peripheral |
| --- | --- | --- |
| PIR Output | D2 | PIR Sensor |
| Status LED | D8 | 3.0 mm LED |
| Alarm LED | D9 | 5.0 mm LED |
| Buzzer | D10 | Piezo Buzzer |

All peripherals are powered from the Arduino R4 WiFi 5 V supply and share a common ground.

Figure 2 – Hardware Wiring Overview
Illustrates GPIO connections between the Arduino R4 WiFi, PIR sensor, LEDs, and buzzer.

### 5.2 Design Considerations

- GPIO current limits must not be exceeded.

- PIR sensor voltage requirements must be verified prior to connection.

- Secure mounting is required to prevent accidental disconnection.

## 6. Software Design

### 6.1 State Machine

Table 4 – System State Definitions

| State | Description |
| --- | --- |
| DISARMED | System inactive; PIR ignored |
| ARMED | PIR monitored; status LED active |
| ALARM_TRIGGERED | Alarm LED and buzzer active |

Figure 3 – System State Diagram
Shows transitions between DISARMED, ARMED, and ALARM_TRIGGERED states.

### 6.2 Web Interface

Table 5 – HTTP Endpoints

| Method | Endpoint | Function |
| --- | --- | --- |
| GET | /status | Returns system state |
| POST | /arm | Arms the system |
| POST | /disarm | Disarms the system |

## 7. Testing and Validation

Table 6 – Test Results Summary

| Test Case | Expected Result | Outcome |
| --- | --- | --- |
| Power-Up | System starts DISARMED | Pass |
| Arm Command | Status LED ON | Pass |
| Motion Detection | Alarm triggered | Pass |
| Disarm Command | Outputs OFF | Pass |

## 8. Cost Estimate

Table 7 – Cost Breakdown (Excluding Board)

| Item | Estimated Cost (USD) |
| --- | --- |
| PIR Sensor | $2–8 |
| LEDs & Buzzer | $3–10 |
| Wiring & Enclosure | $5–10 |
| Total | $10–25 |

## 9. Limitations and Future Work

- Add HTTPS and authentication for secure remote access.

- Integrate event logging and cloud connectivity.

- Expand with camera-based verification and AI-assisted filtering.

- Add battery backup and tamper detection.

## 10. Conclusion

This project demonstrates a formal engineering approach to designing a network-enabled security system using the Arduino R4 WiFi. The system successfully integrates hardware sensing, firmware control logic, and web-based user interaction, making it suitable for educational demonstrations and early-stage professional prototyping.

## Appendix A — Safety Notes

- Operate only within low-voltage DC limits.

- Verify all GPIO current ratings before deployment.

- Use an enclosed housing for long-term operation.